FS China VPN Solutions

Site-to-Site VPN connection scenarios:

- AODC – AWS or AWS CN – AWS Global over DX

- Cross VPCs (namely cross accounts) - needs clarification if encryption between TGWs is enough or end2end encryption is required

Proposals

- Third Party VPN

- Opensource VPN

- AWS Native S2S VPN landing in China

- AWS Native Infra with partial encryption

Architecture

For Option A and B, there are 2 possible designs; it covers scenario1 and scenario2 if encryption between TGWs satisfies the security requirements.

- Third-party VPN or Opensource VPN ends at VPN VPC VGW, the traffic goes through VPN then routes to TGW

- Third-party VPN or Opensource VPN ends at TGW, the traffic routes firstly to TGW and gets processed through attached VPN VPC then back to TGW

For scenario 2 if end2end encryption is required, VPN appliances can be deployed in the customer VPCs. This approach adds complexity to architecture and deployment but is viable.

For Option C and D, the architecture remains the same as FS Global

Option A

Third-Party VPNs like Forti PA and most 3rd party VPNs can be integrated with AWS Transit Gateway.  ECMP can be setup to meet throughput requirements; HA is also supported.

Operation Overhead

When the number of instances is more than 10, it’s recommended to use 3rd party monitoring tool like Forti Manager or PA Hub, otherwise to configure VPN Appliances to send syslog into an existing monitoring system for integration.

Forti or PA provides a support portal for ticket/case submission via email or call. They also offer IM channels like slack groups for setup and support.  They have their SLA once support tier is purchased.

a. Fortinet – Automation setup via IaC with Terraform FortiOS provider

Fortinet can conduct PoC, validating the site2site vpn automation requirement (estimated 1 week) and delivering IaC templates to FS for future deployment.

Fortinet has experience helping customers implement automated Fortinet VM startups and initializations on AWS through Terraform.

Fortinet can also support license pool management and generate on-demand license from the pool which can be integrated into the initialization process.

Fortinet provides FortiManager as a centralized management platform. Forti VM can also be initialized with syslog forwarding during IaC launch if customized integration is needed.

*FortiFlex licensing (license pool) is a points-based, pay-as-you-go consumption model. Instead of buying a static license for a specific hardware serial number, you purchase "Points" that can be used to spin up or scale various services dynamically.

b. Same 3rd party ISV solution that has been used by FS China

Strength: no extra vendor approval process needed

Weakness: the degree of automation remains to be validated

Palo Alto as a reference:

PA still primarily relies on traditional engineers' manually configuring services. Their API maturity and experience in IaC automation scenarios are quite limited.

PA supports license pool but not an auto license import mechanism.

PA provides PA Hub as a centralized management platform.

Option B - Opensource VPN (only when 3rd party ISV cannot meet requirements like automation feature, encryption etc., or cost is a big concern)

OpenSwan / StrongSwan

Strength:  Self-managed EC2, IaC basic automation ready (needs to adapt with TGW) https://github.com/aws-samples/vpn-gateway-strongswan

Weakness:  Self-maintenance / troubleshooting, expected 0.5 person, EC2 instance metrics logging collection and forwarding for integration

SA and network SSA can conduct PoC validating the site2site VPN automation, metrics and logging export etc., estimated 2 weeks

Option C - AWS Native S2S VPN solution landing in China

Strength:  least effort for deployment and operation

Weakness: Timeline is not clear. AWS Legal / Security Team needs to be involved and provide roadmap or explanation

Option D - AWS Native Infra with partial encryption

Scenario1 - Direct Connect without encryption (MACsec N/A in China region)

Scenario2 – Instances with Nitro card encryption ( >c6, m6, r6 in same region, VPC peering + TGW encryption by default when cross China regions)